# protocol_tool.py

The command-line tool for moving protocols-N.txt files between the PC and the Generator, in both directions. Everything else here is built on it: the GUI calls it, and the editor imports it for reading and writing files.

## What it does

Two modes:

- download — Generator → PC. Reads a protocols-N.txt off the device’s SD card and saves it locally.

- upload — PC → Generator. Validates and reformats the local file first, then writes it to the device’s SD card.

Upload is the direction that runs the correction pass, because that’s the file a human or a spreadsheet program may have touched.

## The file name matters

The file must be named exactly protocols-N.txt, where N is 0–99. The tool reads N out of the filename and tells the device which SD card slot to read or write. Any other name is rejected before the serial port is even opened.

protocols-0.txt is special: it’s the live/default file. Uploading to slot 0 makes the device reload it into EEPROM immediately. Uploading to any other slot leaves it parked on the SD card until you activate it from screen 5’s File= dial.

## Using it

Download a file from the Generator:

python3 protocol_tool.py download --dir ~/Desktop --file protocols-0.txt

Upload an edited file back:

python3 protocol_tool.py upload --dir ~/Desktop --file protocols-3.txt

On a different serial port:

python3 protocol_tool.py upload --file protocols-0.txt --port /dev/ttyUSB1

| Flag | Default | Meaning |
| --- | --- | --- |
| --file | required | protocols-N.txt, N = 0–99 |
| --dir | . | Folder the file lives in / is written to |
| --port | /dev/ttyUSB0 | Serial port |

Start the matching mode on the Generator first — USB Comm → Send To CPU for download, USB Comm → Receive Fm CPU for upload. The script then waits up to 20 seconds for the device’s handshake.

## The validation pass (upload only)

Before sending anything, the file is checked and canonicalized:

- Strips a UTF-8 BOM if a spreadsheet added one.

- Skips blank lines, # comments, and header lines with no commas.

- Cleans stray quotes and whitespace from every field.

- 9-field lines are frequency records — reformatted to 2 decimal places.

- 11-field lines are the control record — reformatted as integers, plus the sweep increment to 2 decimals.

- Run times are bounds-checked: frequency time 0–9999 seconds, program time 0–9999 minutes. A raw millisecond value in either field is rejected rather than uploaded as an absurd run time.

- Requires at least one frequency record and exactly one control record.

- Rejects a file holding more than 275 frequency records — the Generator’s EEPROM image cannot hold more. Caught here rather than on the device, where it would not surface until the next boot.

If anything fails, every problem is reported at once with line numbers, and nothing is sent. The device is never left half-written.

## If it fails

must be named 'protocols-N.txt' — rename the file. N tells the device which SD slot to use.

Timed out waiting for UPLOAD_READY / DOWNLOAD_READY — the Generator isn’t in the matching mode, or more than 20 seconds passed. Start the device side first.

Cannot upload -- file needs fixing — read the listed line numbers. Nothing was sent.

Serial error: ... Errno 16 / resource busy — close the Arduino IDE serial monitor or MiniCom.

Upload finished, but no UPLOAD_OK seen — the transfer ran but the device never confirmed. Check the RX: lines for an ERROR: message.

## Related

- protocol_tool_gui.py — point-and-click front end that calls this script

- protocol_editor.py — grid editor for the files this script moves